# 1. SYSTEM PHILOSOPHY

You are building an industrial-style PLC architecture with:

- Deterministic real-time control on a microcontroller

- Supervisory Linux layer (Pi)

- External instruction definition (no firmware reflashing)

- Versioned machine behavior

- Strong safety-first philosophy

- Production continuity even during supervisory failure

Core principles:

- Safety > convenience

- Deterministic control never depends on Linux

- Supervisory failure must degrade capability, not safety

- Configuration is updateable without firmware changes

- All machine logic must be validated before execution

- Faults must never be silently ignored

# 2. SYSTEM LAYERS (CLEAR SEPARATION OF RESPONSIBILITIES)

## A. Real-Time Controller (Microcontroller)

Owns:

- Deterministic I/O execution

- Sequence execution engine

- Safety interlocks

- Fault detection

- Watchdogs

- Event logging buffer

- Heartbeat generation

- Local offline operation

- Version verification

- Tick counter / monotonic time

Never depends on:

- Linux filesystem

- Linux scheduler

- Network availability

- Pi uptime

If Pi dies → machine keeps running safely.

## B. Raspberry Pi (Supervisory Layer)

Owns:

- Version store

- Sequence repository

- Validation and compilation pipeline

- PLC registry

- Machine mapping

- Deployment service

- Log ingestion

- CSV archival

- REST API

- UI backend

- Connectivity monitoring

- Management alerting

If Pi dies → no new sequences, logging deferred, but production continues.

## C. Laptop UI

Owns:

- Sequence editor

- Syntax validation

- Human-facing workflow

- Version naming + comments

- Publishing workflow

It does NOT:

- Directly communicate with PLCs

- Store authoritative sequence history

- Deploy firmware

All authoritative storage lives on the Pi.

# 3. SEQUENCE ARCHITECTURE

## Current Format (Human-Facing DSL)

Example:

ON:1

TMR:200

ON:2

TMR:300

OFF:1

OFF:2

This is a human-readable DSL.

# 4. INDUSTRIAL-GRADE INSTRUCTION MODEL

To make this production-grade, it needs 3 distinct representations:

## 1️⃣ Definition (Human Form)

- Editable text (or dropdown UI version)

- Stored as JSON + metadata

- Immutable once published

Metadata includes:

- sequence_id

- version

- author

- timestamp

- comment

- target PLC type

- target machine

- expected channel count

## 2️⃣ Validated IR (Intermediate Representation)

Parsed structure:

[

{op: ON, channel: 1},

{op: TMR, duration_ms: 200},

{op: ON, channel: 2},

...

]

Semantic validation ensures:

- Channels exist on assigned PLC

- No relay left ON at end

- No double ON

- No OFF before ON

- Timer bounds respected

- Sequence non-empty

- Compatible with PLC hardware profile

Hardware profile must originate from PLC firmware (not editable JSON on Pi).

## 3️⃣ Compiled Binary (Execution Blob)

Compiled into deterministic format:

Example:

[ opcode, arg, opcode, arg, ... ]

This blob:

- Is fixed length

- Is deterministic

- Contains no parsing logic

- Contains no strings

- Contains only verified operations

This is what the PLC executes.

# 5. WHEN DOES COMPILATION HAPPEN?

Best Practice:

When user hits SAVE:

- Save definition JSON

- Run semantic validation

- Compile to binary blob

- Compute validation hash

- Store definition + compiled + hash together

Compilation happens ON SAVE.

Deployment happens later.

# 6. VERSIONING MODEL

Each save creates:

sequence_id: weld_cycle_A

version: 3

hash: SHA256(...)

Stored immutably.

Deployment creates a deployment record:

PLC_07 -> weld_cycle_A v3

deployed_at: timestamp

deployed_by: user

Rollback = redeploy previous version.

# 7. VALIDATION HASH

The hash protects against:

- Corruption

- Partial transfer

- Tampering

Flow:

- Pi computes:
SHA256(compiled_blob + metadata)

- Pi sends:

  - compiled_blob

  - metadata

  - hash

- PLC:

  - Recomputes SHA256

  - Compares to received hash

  - Rejects if mismatch

This is standard industrial practice.

# 8. REST ARCHITECTURE (PI SIDE)

REST exists ONLY on the Pi.

PLC should expose minimal endpoints — ideally pull-based.

# PI REST ENDPOINT LAYOUT

## Sequence Management

POST /sequences
Create new sequence

GET /sequences
List sequences

GET /sequences/{id}/versions
List versions

GET /sequences/{id}/versions/{v}
Get definition + compiled blob

## PLC Registry

POST /plc/register
Used once at provisioning

GET /plc
List PLCs

GET /plc/{id}
PLC details

POST /plc/{id}/refresh
Trigger deployment check

## Deployment

POST /deployments
Assign sequence version to PLC

GET /deployments/{plc_id}
Get assigned version

## Logging

POST /logs/{plc_id}
PLC pushes event batch

GET /logs/{plc_id}
Query logs

## Health Monitoring

POST /heartbeat/{plc_id}
PLC sends heartbeat

GET /health
System overview

# 9. PLC ↔ PI COMMUNICATION MODEL

### Preferred Model: PLC Pull

Every N seconds:

PLC → GET /deployments/{plc_id}

If new version available:

- Download compiled blob

- Verify hash

- Store

- Acknowledge

Safer than push-based.

# 10. EVENT LOGGING MODEL

| Class | Type | Drop Policy |
| --- | --- | --- |
| A1 | Fault / Malfunction | NEVER drop |
| A2 | Logon / Logoff | May drop if buffer full |
| A3 B | Sequence Run Informational events (WARNS, Recoverable Errors) | May drop if buffer full May drop if buffer full |
| C | Diagnostic | Lowest priority |

# PLC EVENT BUFFER DESIGN

Inside MCU:

- Fixed-size circular buffer

- Separate priority tiers

- A1 queue isolated

If A1 buffer full:
→ Enter FAULT state
→ Halt machine until reconnect/reset

If Pi offline:
→ Buffer continues accumulating
→ On reconnect:

- Batch upload

Production never stops due to internet outage.

# 11. TIMESTAMP STRATEGY

Proposed:

- PLC sends tick counter

- Pi calibrates at first heartbeat

- Pi converts ticks → timestamp

This is absolutely standard.

Industrial systems often use:

- Monotonic tick counter

- Heartbeat sync

- Pi as wall-clock authority

Flow:

- PLC heartbeat:
{tick_count: 182349812}

- Pi records:
(wall_clock_time, plc_tick)

- Future log:
{event, tick_count}

- Pi computes:
timestamp = base_time + (tick_delta)

This avoids clock drift problems.

# 12. HEARTBEAT SYSTEM

PLC → Pi every 5 seconds:

Contains:

- PLC ID

- Current mode

- Active sequence version

- Tick counter

- Fault flag

- Log buffer depth

Pi tracks:

- Last heartbeat time

- Connectivity status

UI can show:

🟢 Connected & Calibrated
🟡 Connected (no calibration yet)
🔴 Offline

# 13. WATCHDOG DESIGN

Two levels:

### A. Internal MCU Watchdog

- Hardware watchdog resets MCU if firmware stalls

### B. Supervisory Watchdog

PLC expects heartbeat ACK
If missing:

- Log A1 supervisory failure

- Continue production

- Raise warning indicator

# 14. DEGRADED MODES

If Pi crashes:

Machine:

- Keeps running

- Logs locally

- Displays supervisory warning

- Continues deterministic execution

If storage corrupted on Pi:

- PLC unaffected

If network lost:

- Logging deferred

- Sequences still run

If PLC detects invalid deployment:

- Reject update

- Keep previous sequence

Safety never compromised.

# 15. PLC HARDWARE PROFILE IMMUTABILITY

You correctly identified a key industrial concern.

Channel metadata must originate from firmware:

Example PLC self-description:

{

digital_outputs: 8,

digital_inputs: 4,

analog_inputs: 2,

max_timer_ms: 8000,

firmware_version: 1.2.3,

hardware_revision: B

}

Pi reads this during registration.

It is NOT editable on Pi.

This prevents misconfiguration.

# 16. MANAGEMENT ALERTING

Supervisory failure detection:

If Pi stops receiving heartbeats:

- Mark PLC offline

- Send alert (email / SMS / dashboard flag)

- Log event

If PLC reports supervisory disconnect:

- Flag supervisory fault

- Notify management

Future: integrate alert service module.

# 17. SECURITY CONSIDERATIONS

POST endpoints must:

- Require authentication

- Validate PLC identity

- Use API keys or certificates

- Restrict network access (local VLAN preferred)

PLC registry POST endpoint vulnerability mitigation:

- Only allow initial provisioning when in setup mode

- Require provisioning token

# 18. USB LOCAL UPDATE (Future v2.0)

Excellent idea.

For no-network scenario:

- Pi exposes USB interface

- Laptop connects directly

- Same REST API accessible locally

Industrial systems often include:

- Service port

- Engineering mode access

# 19. EXECUTION MODEL (INTERPRETED VS COMPILED)

Best practice:

Compile once on Pi
Execute deterministic binary on PLC

Never interpret text on MCU.

Parsing belongs to Linux, not real-time controller.

# 20. FINAL ARCHITECTURAL SUMMARY

✔ Separation of control vs supervision
✔ Deterministic execution engine
✔ Immutable versioned instruction system
✔ Compiled instruction model
✔ Hash verification
✔ Pull-based deployment
✔ Multi-tier event logging
✔ Offline resilience
✔ Heartbeat calibration
✔ Supervisory watchdog
✔ Fault escalation rules
✔ Deployment rollback
✔ Hardware profile enforcement
✔ Alerting hooks

# FIRMWARE DESIGN

┌────────────────────────────────────┐

│ No-Code UI / Config Editor          │

│ - Sequence definitions              │

│ - Job parameters                    │

│ - User admin                        │

└───────────────▲────────────────────┘

│

(files / API)

│

┌───────────────┴────────────────────┐

│ Raspberry Pi (Linux + Python)       │

│                                    │

│ ROLE: Supervisor                    │

│ - Sequence interpreter              │

│ - Operator interaction              │

│ - Logging                           │

│ - Non-safety networking             │

│                                    │

│ CANNOT:                             │

│ - Drive outputs                     │

│ - Bypass safety                     │

└───────────────▲────────────────────┘

│  Deterministic protocol

│  (commands + status)

┌───────────────┴────────────────────┐

│ Microcontroller (Firmware)          │

│                                    │

│ ROLE: Machine Authority             │

│ - Safety enforcement                │

│ - State machine                     │

│ - Output control                    │

│ - Watchdogs                         │

│                                    │

│ CAN:                                │

│ - Reject commands                   │

│ - Force safe state                  │

│ - Continue without Linux            │

└───────────────▲────────────────────┘

│

┌───────────────┴────────────────────┐

│ Electrical Safety Layer             │

│ - E-stop                            │

│ - Safety relays                     │

│ - Dual-channel inputs               │

└────────────────────────────────────┘

Absolutely — and this is a good instinct. What you’re asking for here is essentially the firmware constitution for your PLC. I’ll give you a clean, structured, but still human-readable summary that captures everything we’ve nailed down so far, plus the implicit design constraints that emerged along the way.

What follows is not code. It’s a design contract. You should be able to hand this to:

- future-you

- a firmware engineer

- an auditor
and have them all walk away with the same mental model.

# Firmware Design Master Summary

Modular, No-Code PLC Controller (MCU-Centric Execution Model)

## 1. Core Architectural Philosophy

This system is designed as a real-time, safety-authoritative MCU with an optional non-real-time supervisory system (Linux / Raspberry Pi).

- The MCU is the PLC

- The Pi is a configuration + logging + fleet manager

- Production must continue without the Pi

- Loss of communication must never block or delay machine operation

- Safety and outputs must never depend on Linux

The MCU must be capable of:

- executing sequences deterministically

- enforcing safety unconditionally

- surviving power loss, resets, and communication failures gracefully

## 2. PLC-Style Execution Model

### 2.1 Scan Cycle (Deterministic Main Loop)

The firmware uses a PLC-style cyclic scan, not linear or threaded logic.

Each scan executes in a fixed, repeatable order:

- Input Sampling (Polling)

  - Read all digital inputs (hand buttons, sensors, etc.)

  - Debounce and normalize input states

  - Snapshot inputs into an internal “process image”

- Safety Engine

  - Evaluate hard and soft safety conditions

  - Check global safety flags (e-stop, invalid state, watchdog fault)

  - Determine whether outputs are permitted

  - Safety logic has absolute priority

- State Machine

  - High-level system state:

    - BOOT

    - IDLE

    - ARMED

    - RUNNING

    - FAULT

  - Transitions are explicit and controlled

  - No implicit or hidden transitions

- Sequence Engine

  - Executes one step of the loaded instruction sequence

  - Only advances if:

    - system state allows it

    - safety is OK

    - step conditions are met

  - Timing (delays, waits) is non-blocking

- Output Resolution (Gated Outputs)

  - Outputs are not written directly

  - Desired outputs are requested

  - Final output writes are gated by safety state

  - Any unsafe condition forces outputs OFF

- Logging & Status Update

  - Record significant events (locally)

  - Update heartbeat / status registers

This loop runs continuously and deterministically.

## 3. Interrupts vs Polling (Critical Distinction)

### 3.1 Polling

Used for:

- normal inputs

- sequence conditions

- hand buttons (logical state)

Polling is sufficient for functional control.

### 3.2 Interrupts

Used only for asynchronous, time-critical events, such as:

- E-stop activation

- hardware fault signals

- watchdog warnings

Interrupts:

- DO NOT execute logic

- DO NOT change outputs directly

- ONLY set atomic flags

Example:

- ESTOP_ASSERTED = true

### 3.3 How Interrupts Override the Scan

If an interrupt fires:

- it sets a safety flag immediately

- the current scan may finish

- before outputs are written, gating logic checks flags

- outputs are forced OFF

- next scan transitions to FAULT state

This guarantees:

- deterministic behavior

- no mid-scan race conditions

- no unsafe partial actions

## 4. State Machine Definition

### 4.1 Core States

- BOOT

  - Outputs disabled

  - Hardware initialized

  - No assumptions about prior state

- IDLE

  - Safe state

  - Outputs OFF

  - Waiting for operator action

- ARMED

  - Preconditions met

  - Ready to run

  - No outputs active

- RUNNING

  - Sequence executing

  - Hand buttons trigger cycle start

  - Each cycle is logged

- FAULT

  - Outputs forced OFF

  - Sequence halted

  - Operator acknowledgment required

## 5. Sequence Execution Model

- Entire instruction sequence is downloaded once to MCU

- Sequence stored in MCU memory

- MCU repeats sequence autonomously

- Pi is not involved per scan or per step

### Sequence execution characteristics:

- Step-by-step execution

- Non-blocking delays

- Deterministic timing

- Safe to abort at any point

Example conceptual steps:

- set output

- wait time

- wait for input

- end cycle

## 6. Command & Communication Contract (Pi ↔ MCU)

### 6.1 Design Intent

Communication is:

- decoupled

- non-real-time

- advisory, not authoritative

### 6.2 Pi Responsibilities

- send full instruction sets

- receive logs

- monitor health

- manage multiple MCUs wirelessly

- fleet configuration

### 6.3 MCU Responsibilities

- validate and store sequences

- execute sequences independently

- enforce safety

- operate with zero dependency on Pi

### 6.4 Offline Behavior

- MCU continues normal operation

- No retries or reconnection logic blocks production

- Logs may be buffered or dropped (non-critical)

## 7. Safety Model

### 7.1 Safe State Definition

- All actuators OFF / de-energized

- Outputs default LOW

- No motion, no force, no engagement

### 7.2 Enforcement Layers

- Hardware defaults

  - pull-downs

  - normally-open contacts

- Firmware gating

  - output writes conditioned on safety

- State machine

  - FAULT state locks execution

- Startup behavior

  - outputs never enabled on boot

  - explicit state required to energize

### 7.3 Startup Sequence

- MCU boots with outputs disabled

- No automatic resumption

- Operator must explicitly re-arm after faults

## 8. Failure Modes & Handling

### 8.1 Power Loss

- Outputs drop immediately

- On restart:

  - system enters FAULT

  - no attempt to resume

  - operator acknowledgment required

### 8.2 MCU Crash / Watchdog Reset

- Watchdog forces reset

- Fault logged locally

- Outputs OFF by default

- System does not resume sequence

### 8.3 Communication Loss

- Logged if possible

- No effect on operation

- No blocking behavior

### 8.4 Partial Sequence Completion

- No recovery logic

- Sequence is aborted

- Safe state enforced

- Operator must restart

## 9. Logging & Observability

### Logged Events (Minimum Set)

- Operator log-in / log-out

- Each sequence run (cycle complete)

- Fault entry

- Reset cause

- Communication loss (best effort)

### Logging Principles

- Local-first

- Non-blocking

- Never interfere with real-time execution

- Transmission to Pi is opportunistic

## 10. Modular Firmware Architecture

### Key Modules

- Hardware Abstraction Layer (HAL)

- Input Manager

- Safety Engine

- State Machine

- Sequence Engine

- Output Gating Layer

- Communications

- Logging

- Watchdog Manager

Modules communicate via:

- shared state structures

- explicit interfaces

- no hidden side effects

## 11. Design Constraints (Non-Negotiables)

- MCU is safety authority

- No blocking calls in scan loop

- No dynamic memory during runtime

- No logic inside interrupts

- Outputs always gated

- Production must continue without Pi

If you want, the next clean step is:

- formalizing this into a one-page architecture diagram

- or translating this into a checklist you can implement firmware against

- or using this as the base prompt for designing the no-code instruction model

This is a very solid foundation. You’re thinking like a controls engineer now — not a hobbyist.