┌────────────────────────────────────┐

│ No-Code UI / Config Editor          │

│ - Sequence definitions              │

│ - Job parameters                    │

│ - User admin                        │

└───────────────▲────────────────────┘

│

(files / API)

│

┌───────────────┴────────────────────┐

│ Raspberry Pi (Linux + Python)       │

│                                    │

│ ROLE: Supervisor                    │

│ - Sequence interpreter              │

│ - Operator interaction              │

│ - Logging                           │

│ - Non-safety networking             │

│                                    │

│ CANNOT:                             │

│ - Drive outputs                     │

│ - Bypass safety                     │

└───────────────▲────────────────────┘

│  Deterministic protocol

│  (commands + status)

┌───────────────┴────────────────────┐

│ Microcontroller (Firmware)          │

│                                    │

│ ROLE: Machine Authority             │

│ - Safety enforcement                │

│ - State machine                     │

│ - Output control                    │

│ - Watchdogs                         │

│                                    │

│ CAN:                                │

│ - Reject commands                   │

│ - Force safe state                  │

│ - Continue without Linux            │

└───────────────▲────────────────────┘

│

┌───────────────┴────────────────────┐

│ Electrical Safety Layer             │

│ - E-stop                            │

│ - Safety relays                     │

│ - Dual-channel inputs               │

└────────────────────────────────────┘

Absolutely — and this is a good instinct. What you’re asking for here is essentially the firmware constitution for your PLC. I’ll give you a clean, structured, but still human-readable summary that captures everything we’ve nailed down so far, plus the implicit design constraints that emerged along the way.

What follows is not code. It’s a design contract. You should be able to hand this to:

- future-you

- a firmware engineer

- an auditor
and have them all walk away with the same mental model.

# Firmware Design Master Summary

Modular, No-Code PLC Controller (MCU-Centric Execution Model)

## 1. Core Architectural Philosophy

This system is designed as a real-time, safety-authoritative MCU with an optional non-real-time supervisory system (Linux / Raspberry Pi).

- The MCU is the PLC

- The Pi is a configuration + logging + fleet manager

- Production must continue without the Pi

- Loss of communication must never block or delay machine operation

- Safety and outputs must never depend on Linux

The MCU must be capable of:

- executing sequences deterministically

- enforcing safety unconditionally

- surviving power loss, resets, and communication failures gracefully

## 2. PLC-Style Execution Model

### 2.1 Scan Cycle (Deterministic Main Loop)

The firmware uses a PLC-style cyclic scan, not linear or threaded logic.

Each scan executes in a fixed, repeatable order:

- Input Sampling (Polling)

  - Read all digital inputs (hand buttons, sensors, etc.)

  - Debounce and normalize input states

  - Snapshot inputs into an internal “process image”

- Safety Engine

  - Evaluate hard and soft safety conditions

  - Check global safety flags (e-stop, invalid state, watchdog fault)

  - Determine whether outputs are permitted

  - Safety logic has absolute priority

- State Machine

  - High-level system state:

    - BOOT

    - IDLE

    - ARMED

    - RUNNING

    - FAULT

  - Transitions are explicit and controlled

  - No implicit or hidden transitions

- Sequence Engine

  - Executes one step of the loaded instruction sequence

  - Only advances if:

    - system state allows it

    - safety is OK

    - step conditions are met

  - Timing (delays, waits) is non-blocking

- Output Resolution (Gated Outputs)

  - Outputs are not written directly

  - Desired outputs are requested

  - Final output writes are gated by safety state

  - Any unsafe condition forces outputs OFF

- Logging & Status Update

  - Record significant events (locally)

  - Update heartbeat / status registers

This loop runs continuously and deterministically.

## 3. Interrupts vs Polling (Critical Distinction)

### 3.1 Polling

Used for:

- normal inputs

- sequence conditions

- hand buttons (logical state)

Polling is sufficient for functional control.

### 3.2 Interrupts

Used only for asynchronous, time-critical events, such as:

- E-stop activation

- hardware fault signals

- watchdog warnings

Interrupts:

- DO NOT execute logic

- DO NOT change outputs directly

- ONLY set atomic flags

Example:

- ESTOP_ASSERTED = true

### 3.3 How Interrupts Override the Scan

If an interrupt fires:

- it sets a safety flag immediately

- the current scan may finish

- before outputs are written, gating logic checks flags

- outputs are forced OFF

- next scan transitions to FAULT state

This guarantees:

- deterministic behavior

- no mid-scan race conditions

- no unsafe partial actions

## 4. State Machine Definition

### 4.1 Core States

- BOOT

  - Outputs disabled

  - Hardware initialized

  - No assumptions about prior state

- IDLE

  - Safe state

  - Outputs OFF

  - Waiting for operator action

- ARMED

  - Preconditions met

  - Ready to run

  - No outputs active

- RUNNING

  - Sequence executing

  - Hand buttons trigger cycle start

  - Each cycle is logged

- FAULT

  - Outputs forced OFF

  - Sequence halted

  - Operator acknowledgment required

## 5. Sequence Execution Model

- Entire instruction sequence is downloaded once to MCU

- Sequence stored in MCU memory

- MCU repeats sequence autonomously

- Pi is not involved per scan or per step

### Sequence execution characteristics:

- Step-by-step execution

- Non-blocking delays

- Deterministic timing

- Safe to abort at any point

Example conceptual steps:

- set output

- wait time

- wait for input

- end cycle

## 6. Command & Communication Contract (Pi ↔ MCU)

### 6.1 Design Intent

Communication is:

- decoupled

- non-real-time

- advisory, not authoritative

### 6.2 Pi Responsibilities

- send full instruction sets

- receive logs

- monitor health

- manage multiple MCUs wirelessly

- fleet configuration

### 6.3 MCU Responsibilities

- validate and store sequences

- execute sequences independently

- enforce safety

- operate with zero dependency on Pi

### 6.4 Offline Behavior

- MCU continues normal operation

- No retries or reconnection logic blocks production

- Logs may be buffered or dropped (non-critical)

## 7. Safety Model

### 7.1 Safe State Definition

- All actuators OFF / de-energized

- Outputs default LOW

- No motion, no force, no engagement

### 7.2 Enforcement Layers

- Hardware defaults

  - pull-downs

  - normally-open contacts

- Firmware gating

  - output writes conditioned on safety

- State machine

  - FAULT state locks execution

- Startup behavior

  - outputs never enabled on boot

  - explicit state required to energize

### 7.3 Startup Sequence

- MCU boots with outputs disabled

- No automatic resumption

- Operator must explicitly re-arm after faults

## 8. Failure Modes & Handling

### 8.1 Power Loss

- Outputs drop immediately

- On restart:

  - system enters FAULT

  - no attempt to resume

  - operator acknowledgment required

### 8.2 MCU Crash / Watchdog Reset

- Watchdog forces reset

- Fault logged locally

- Outputs OFF by default

- System does not resume sequence

### 8.3 Communication Loss

- Logged if possible

- No effect on operation

- No blocking behavior

### 8.4 Partial Sequence Completion

- No recovery logic

- Sequence is aborted

- Safe state enforced

- Operator must restart

## 9. Logging & Observability

### Logged Events (Minimum Set)

- Operator log-in / log-out

- Each sequence run (cycle complete)

- Fault entry

- Reset cause

- Communication loss (best effort)

### Logging Principles

- Local-first

- Non-blocking

- Never interfere with real-time execution

- Transmission to Pi is opportunistic

## 10. Modular Firmware Architecture

### Key Modules

- Hardware Abstraction Layer (HAL)

- Input Manager

- Safety Engine

- State Machine

- Sequence Engine

- Output Gating Layer

- Communications

- Logging

- Watchdog Manager

Modules communicate via:

- shared state structures

- explicit interfaces

- no hidden side effects

## 11. Design Constraints (Non-Negotiables)

- MCU is safety authority

- No blocking calls in scan loop

- No dynamic memory during runtime

- No logic inside interrupts

- Outputs always gated

- Production must continue without Pi

If you want, the next clean step is:

- formalizing this into a one-page architecture diagram

- or translating this into a checklist you can implement firmware against

- or using this as the base prompt for designing the no-code instruction model

This is a very solid foundation. You’re thinking like a controls engineer now — not a hobbyist.